Project Progress Update

So far, we have successfully formed and joined the network and enabled the transmission of ON/OFF button commands between the Light and Switch devices. Now, we will extend the project further by integrating the RHT sensor available on the board. The goal is to transmit sensor data from the router to the coordinator.

Note:
 This document does not cover the steps for network creation/forming or sending ON/OFF commands. It begins directly with sensor integration. Please refer to earlier documents for those initial steps.

Switch Side  : 

Now we try to install the below software components on Switch side 


1) Identify Feedback

2) Reporting

3) Relative Humidity and Temperature Sensor

4) Find and Bind Target

Now Lets configure the ZCL Advanced Platform (ZAP) on switch side 


1) In the existing endpoint, in the measurement and sensing section we can find the Temperature Measurement and set it as client as shown below 


2) Now go to the setting button and enable reporting status as shown below

Light:

Now Lets configure the ZCL Advanced Platform (ZAP) on Light side 

1)   In the existing endpoint, in the measurement and sensing section we can find the Temperature Measurement and set it as server as shown below

And within the settings section enable the reporting status : 




Now we try to install the below software components on Light side

1) Reporting

2) Find and Bind Target

Code (Switch)

Please copy the relevant code from the GitHub repository linked below:

https://github.com/VardhanN10/Wireless-Sensor-Network-using-Zigbee/blob/main/Router/app/app.c


Code (Light)

Please copy the relevant code from the GitHub repository linked below:

https://github.com/VardhanN10/Wireless-Sensor-Network-using-Zigbee/blob/main/Coordinator/app/app.c

Workflow: 
On Light side :

1)Network Formation 

	plugin network-creator start 1

plugin network-creator-security open-network

2) Find and binding the target endpoint:

	plugin find_and_bind target 1

On Switch side :

We have written code such that joining the network is done by pressing the button 0 on the board and enabling the reporting sensor data is done by pressing button 1 on the board.





On Light side we get:

(OPTIONAL)

Instead of using an existing endpoint we can create a custom endpoint, here we can configure the endpoints according to our needs.

On Light Side:

As It is responsible for formation of the network, identifying the devices which are joining, we have to enable Basic, Identify, Groups and Scenes as a server(as it is acting as a Coordinator).
But in our project we are also dealing with ON/OFF configuration, so we have to enable ON/OFF as a server as shown below.

 

Now the general configuration is done, we have to enable the temperature measurement as a server and enable reporting in settings as well.

On Switch Side:

Here we only need basic(client), identify(client and server) and ON/OFF(client) in general configuration and in measurement and sensing we need temperature measurement as a client and also need to enable the reporting .

Now by building the project and flashing it into the board by running the commands mentioned in the workflow we are able to send the temperature from client to the server.



Joining Additional Router to the Zigbee Network.

Initially we used 1 Coordinator and 1 Router. Now we will use an additional router in the network which makes the coordinator react to the commands from both the devices.

- Connect the new device and create the project with Zigbee Minimal Configuration.

- Repeat the steps of the previous documents which we followed for the router ( i.e: Endpoints, Software Components).

- Add the router code, build the project and flash the.s37 file into the new device.

- Check whether install code is derived or not. If not derived, create a batch file and run it with the code given in Forming and Joining Document.

- Now we create a network on Coordinator and from the routers joining the network is done by pressing the button 0 on the board and enabling the reporting sensor data is done by pressing button 1 on the board.